2.4-inch TFT 240370PQ
Wiring test program:
Code test environment: JME-2 core board +1 T STC microcontroller instruction cycles (51 core STC12LE5A60S2) +33 M crystal operating voltage 3.3V MCU
Program default IO connections:

Line of Control: RS-P3 ^ 5; WR-P3 ^ 6; RD-P3 ^ 7; CS-P1 ^ 0;REST-P1 ^ 2;
Cable: DB0-DB7 turn connected P0 ^ 0-P0 ^ 7; DB8-DB15 connection in order
P2 ^ 0-P2 ^ 7; (8-bit mode cannot connect DB0-DB7)
Touch features connection: (do not use the touch do not connect)
D_CLK-P1 ^ 7; D_CS-P1 ^ 4; D_DIN-P3 ^ 0; D_OUT-P3 ^ 1; D_PENIRQ-P3 ^ 4;

● TFT Power supply: power supply for the screen, 2.8-3.3V; remember not to use the 5V 
(With PCB of the module to have inherited the 3V regulator IC, you can enter 5V)
● Backlight Power (LED_A pin): 3.2V maximum backlight power supply (Under the 3.3V power supply current limiting resistor in series 20 ohm or 200 ohm resistor in series under 5V.)

● Compatible with the TFT 8 / 16 bit data interface. Row line switching mode by R1, R2 to achieve, 0 ohm resistors R1 short the 16-bit mode, short the R2 for the 8-bit mode, 8-bit mode, so that High 8 bits (i.e. DB8-DB15). Default shipping is 16-bit interface, if you need 8-bit interface Please notify the owner prior to shipment, or need to make changes (if it is to buy the module, you need 8 Bit interface ahead of the best shows that he changed a lot of trouble. IO 8-bit interface takes more than 16-bit access IO port takes less eight, but more slowly, more than 16-bit interface takes IO 8, but is faster.)

● Data port level: in theory, cannot let the data port level exceeds 3.3V, 5V If you must use
Microcontroller data bus IO connector, because tft internal voltage clamp, using a can,
But it is not always standardized. Experiment for sample tests, the product batch to do when
to improve product stability, better use of low-voltage microcontroller or add a level switch
For IC, or use a resistor divider to the IO.
(AVR's IO capability and high output, TFT internal data bus voltage clamping capability
limited when using the AVR-driven Be sure to use the 3.3V power supply to the microcontroller. If a 5V power supply set to use AVR, please add the level converter IC)